What the software is for

When a customer gives us a color to develop, we scan it using our spectrophotometer and get its L*a*b* (LAB) values. The software should use these LAB values to help us find the best recipe to produce that color.

---

Our Data

We have two types of data to feed the system:

1. Pigment Library — We use Sudarshan brand pigments. We have LAB values for each pigment measured in PP (full tone + tint). This is the base reference for all color calculations.

2. Recipe Database — All our previously developed products, each with:

- Pigment combination used

- Raw material / polymer base

- Final measured LAB value in that polymer

---

What the software should do — Step by Step

Step 1 — User enters the target LAB + selects Polymer (mandatory) and optional filters:

- Application (Pipe / Film / Moulding / Cable / Shoe / Engineering)

- Light Fastness, Weather Fastness, Heat Stability (default 200°C)

- RAL / Pantone reference

- Compliance (Non-Restricted / RoHS 1 / RoHS 2/REACH)

Step 2 — Software filters which pigments are eligible based on the above.

Step 3 — Software searches the recipe database and shows existing recipes with closest LAB values in the selected polymer.

Step 4 — If a close match exists in a different polymer, software suggests what recipe adjustments may achieve the target LAB in the required polymer.

Step 5 — If no match exists, software suggests possible pigment combinations that could achieve the target LAB.

Important Technical Note — Color Mixing Model

Pigment LAB values are not simply additive. The software must use the Kubelka-Munk (K-M) optical model to predict LAB of multi-pigment mixtures. Our tint tone data (pigment:TiO₂ = 1:10) provides the K and S coefficients needed for this.

---

Learning Over Time

The same pigment recipe gives different LAB values in different polymers. Every time our lab makes a batch and measures the result, that data gets added to the system. Over time, the software learns the LAB shift patterns per pigment per polymer and improves its suggestions automatically.

---

In Summary — 4 Core Modules

1. Pigment Library (LAB values + properties per pigment)

2. Recipe Database (all developed products with LAB per polymer)

3. Smart Search & Filter Engine (find closest match based on LAB + constraints)

4. Learning Model (improve cross-polymer predictions from new lab data)